Guidance on WS90 firmware upgrade

1. Double-click "dfuse_demo_v3.0.6_setup.exe" to install the software. After the 	installation is completed, open the installation path, select and install the 	corresponding USB driver of STM32 according to the computer system, and 	various driver paths are shown in the following figure:

2. Connect the WS90 Device to computer with USB cable, and press the reset 	button to restart the Device. Make sure the USB port on your PC is USB2.0. The hardware can’t be recognized if USB3.0 port is used.  The blue LED light flashes, and check whether STM32 Device is 	recognized in the Device manager of the computer, and "STM Device in DFU Mode" shows 	that the driver is normal. If not, please reinstall USB driver, and if still have problem, please contact support@ecowitt.com for further help.

3.Open the installation path, double-click "DfuSeDemo.exe" to run the software;

4.Click the "Choose..." and choose the “WS90_V1.1.7_.dfu” file for upload, then check the "Verify after download" option and click "Upgrade" to Upgrade the firmware.

Ws90_v1.1.7_.dfu

5.The interface for completion of upgrade is as follows. Click "Leave DFU mode" and the 	program will jump to complete upgrade. Press “reset” button to restart the sensor and make sure the LED flashes every 8s.

Troubleshooting

- Q: Unknown USB device

- A: Please install USB driver from Firmware & Tookit.zip